We address small-object misses and unstable localization in YOLOv8n under a 1024 input setting. We propose YOLO-SOD, a lightweight small-object-enhanced variant, and test it on COCO and VisDrone. YOLO-SOD improves COCO AP_S from 36.2 to 41.9 (+5.7) and overall mAP@0.5:0.95 from 47.8 to 49.0 (+1.2). On VisDrone, missed detections drop by 10.4% in dense scenes. This shows the method mainly boosts small-object performance and stays reliable under clutter and occlusion.

The improvements come from three parts that work together: preserving shallow details to keep tiny-target cues, using lightweight multi-scale fusion with position-sensitive attention to enhance cross-scale context and suppress background noise, and applying a small-object-aware objective (reweighting + scale/corner-sensitive regression) to make tiny-box errors produce stronger gradients. Ablations indicate these components add consistent gains.

The cost is moderate and controlled: parameters increase from 3.15M to 3.42M, FLOPs from 92.0G to 98.6G, and FPS changes from 42 to 39 at 1024 resolution. YOLO-SOD is a good fit when small-object recall matters more than a small speed drop. Next, we should optimize 1024 deployment (ONNX/TensorRT, pruning/quantization/distillation), improve crowded-scene assignment stability, and test cross-domain robustness on real data and degraded conditions (low light, haze, motion blur).